Questionnaire for Evaluating OHL Cost Prediction Indicators and the Expert Scoring and Indicator Screening Process

## 1. Development of the Initial Indicator System

To characterize the cost formation mechanism of OHL projects as comprehensively as possible, this study develops an initial indicator system based on a systematic literature review and interviews with engineering practitioners. The system follows the principles of comprehensiveness, scientific validity, and practical operability. According to the role of each indicator in the cost generation process of OHL projects, the initial indicators are classified into five dimensions: temporary works, civil works, equipment and materials, construction techniques, and project attributes. Based on this classification, 20 candidate indicators are identified. These indicators cover site grading, earth excavation and backfilling, bored pile hole forming, foundation steel, concrete, steel tower materials, conductor and OPGW erection length, material prices, voltage level, and geological and climatic conditions.

Table 1 Initial Indicators

| Category | Initial indicator |
| --- | --- |
| Temporary works | Site grading (m2) |
| Civil works | Earth excavation (m3) |
|  | Bored pile drilling length (m) |
|  | Foundation steel quantity (t) |
|  | Concrete volume (m3) |
| Equipment and materials | Tower spacing (m) |
|  | Tower steel quantity(t) |
|  | Steel price (CNY/t) |
|  | Number of tower nameplates |
|  | OPGW unit price (CNY/km) |
|  | OPGW erection length (km) |
|  | Conductor unit price (CNY/t) |
|  | Conductor erection length (km) |
|  | Conductor cross sectional area (mm²) |
| Construction techniques | Number of crossings |
|  | Crossing method |
|  | Tension insulator strings |
| Project attributes | Voltage level (kV) |
|  | Geological conditions |
|  | Climatic characteristics |

## 2. Questionnaire Survey

An expert review method was further used to screen the indicator system. This step was conducted to improve the quantifiability, practical operability, and predictive relevance of the variables used in the subsequent modeling process.

A total of 15 experts with backgrounds in OHL project management, planning and design, cost estimation, or engineering consulting were invited to participate in the scoring process. Among them, five experts were from universities and research institutions. The remaining ten experts were from power grid enterprises, electric power design institutes, or engineering consulting organizations.

The questionnaire is presented as follows:

Dear Madam/Sir,

Thank you for participating in this questionnaire survey on the evaluation of cost prediction indicators for overhead transmission line (OHL) projects. This survey aims to assess the key indicators that affect the cost of OHL projects. The questionnaire is used only for academic research. Your responses will be kept strictly confidential, and your privacy will be protected. Please answer the questions carefully based on your practical experience and understanding. The questionnaire is expected to take about five minutes to complete.

I. Basic Information

Expert ID: ________

Years of professional experience:□ 0–5 years □ 6–10 years □ 11–15 years □ 16–20 years □ More than 20 years

Main professional role: □ Design □ Cost estimation/budgeting □ Construction □ Supervision □ Owner/investor □ Other: _____

Number of transmission line projects participated in: □ 1–5 □ 6–10 □ 11–20 □ More than 20

Main voltage-level experience: □ 110kV □ 220kV □ 500kV □ Other: _____

Region/typical terrain conditions: □ Plain □ Hilly area □ Mountainous area □ High-cold/icing area □ Coastal area with strong winds □ Other: _____

II. Scoring Rules (One-Dimensional Overall Applicability Rating)

Please score each indicator on a scale of 1 to 5, or select NA:

5 = Highly applicable. The indicator can be quantified at an early stage, is easy to obtain, provides substantial additional information, and is critical for cost prediction.

4 = Relatively applicable. The indicator is generally important, although there may be minor limitations in definition or data availability.

3 = Moderate. The indicator has some value, but it may be highly replaceable or unstable in terms of data availability.

2 = Weak. The indicator provides limited additional information, has an unclear definition, or is difficult to obtain at an early stage.

1 = Not applicable. The indicator is difficult to quantify, difficult to obtain, or highly redundant.

NA = Unable to judge or lacking relevant experience.

Please rate the following 20 indicators based on your experience and the principle of their influence on cost prediction.

| Initial indicator | Description | Score |
| --- | --- | --- |
| Site grading | Total area of earthwork leveling in the tower foundation area and temporary steel plate laying for construction. |  |
| Earth excavation | Total volume of earthwork excavated and backfilled during the construction of tower foundations and related facilities. |  |
| Bored pile drilling length | Hole depth from the ground surface to the designed pile-tip elevation during bored pile foundation construction. |  |
| Foundation steel quantity | Total weight of steel used for tower foundations, such as reinforcement cages and reinforcement frameworks. |  |
| Concrete volume | Total volume of concrete used for tower foundations, foundation cushions, and ancillary structures. |  |
| Tower spacing | Average horizontal distance between two adjacent steel towers, namely the span length. |  |
| Tower steel quantity | Total weight of steel used for steel towers along the transmission line. |  |
| Steel price | Market unit price of steel used in the project. |  |
| Number of tower nameplates | Total number of sign plates used to identify tower information. |  |
| OPGW unit price | Market price of OPGW cable per kilometer. |  |
| OPGW erection length | Total erection length of OPGW cable. |  |
| Conductor unit price | Market unit price of overhead transmission conductors. |  |
| Conductor erection length | Total conductor erection length, considering span length, sag, and related factors. |  |
| Conductor cross sectional area | Cross-sectional area of the conductor. |  |
| Number of crossings | Number of crossings over existing facilities, natural obstacles, or man-made structures. |  |
| Crossing method | Construction technique or technical measure adopted for crossing sections. |  |
| Tension insulator strings | Number of hardware assemblies installed on tension towers to withstand conductor tension. |  |
| Voltage level | Rated voltage level of the transmission line. |  |
| Geological conditions | Geological conditions along the transmission line, including lithology, soil type, groundwater level, and foundation bearing capacity. The scoring criteria are: 1 = good soil layer; 2 = general conditions; 3 = soft soil or high groundwater level; 4 = rock or unfavorable geological conditions; 5 = complex and unfavorable geological conditions. |  |
| Climatic characteristics | Meteorological factors along the transmission line, such as temperature, wind speed, ice thickness, and rainfall. This indicator can be classified by icing or wind-speed zones, or according to design code categories. |  |

## 3. Result Processing

Table 2 Expert Scoring Results

| Evaluation indicator | Expert 1 | Expert 2 | Expert 3 | … | Expert 13 | Expert 14 | Expert 15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage level | 5 | 4 | 5 | … | 4 | 5 | 5 |
| Concrete volume | 5 | 4 | 4 | … | 4 | 4 | 5 |
| Foundation steel quantity | 5 | 2 | 4 | … | 5 | 4 | 4 |
| Tower steel quantity | 4 | 4 | 4 | … | 4 | 4 | 4 |
| Steel price | 5 | 5 | 4 | … | 4 | 4 | 4 |
| Conductor cross sectional area | 5 | 4 | 4 | … | 2 | 4 | 3 |
| Conductor erection length | 4 | 4 | 4 | … | 4 | 4 | 3 |
| Bored pile drilling length | 3 | 3 | 4 | … | 4 | 3 | 4 |
| Conductor unit price | 4 | 5 | 4 | … | 5 | 3 | 3 |
| Number of crossings | 3 | 2 | 5 | … | 5 | 4 | 2 |
| OPGW unit price | 2 | 3 | 3 | … | 3 | 3 | 3 |
| Earth excavation | 2 | 4 | 5 | … | 3 | 4 | 3 |
| Site grading | 4 | 4 | 3 | … | 3 | 3 | 2 |
| OPGW erection length | 4 | 3 | 4 | … | 3 | 3 | 5 |
| Number of tower nameplates | 5 | 3 | 3 | … | 3 | 3 | 3 |
| Tension insulator strings | 2 | 4 | 4 | … | 2 | 2 | 4 |
| Geological conditions | 2 | 3 | 2 | … | 2 | 2 | 3 |
| Climatic characteristics | 2 | 3 | 2 | … | 2 | 1 | 3 |
| Tower spacing | 2 | 3 | 2 | … | 2 | 3 | 4 |
| Crossing method | 2 | 3 | 3 | … | 2 | 3 | 2 |

### 3.1 Mean, Standard Deviation, and Coefficient of Variation

For the (j)-th indicator, the mean score: the mean score μj is calculated as follows:

μj=1Ni=1Nsij

The standard deviation σj is calculated as follows:

σj=1N-1i=1NSij-μj2

The coefficient of variation CVj is calculated as follows:

CVj=σjμj

To ensure comparability, the mean score and standard deviation are range-normalized before calculating the composite score:

μj'=μj-minμmaxμ-minμ

For the coefficient of variation, a smaller value indicates stronger agreement among experts. Therefore, in addition to normalization, a reverse transformation is applied:

CVj'=1-CVj-min⁡(CV)maxCV-min⁡(CV)

Table 3 Indicator Scoring Analysis Results

| Evaluation indicator | Mean score | Standard deviation | Coefficient of variation | Composite score |
| --- | --- | --- | --- | --- |
| Voltage level | 4.53 | 0.52 | 0.11 | 1.00 |
| Concrete volume | 4.33 | 0.62 | 0.14 | 0.91 |
| Foundation steel quantity | 4.27 | 0.80 | 0.19 | 0.85 |
| Tower steel quantity | 4.13 | 0.64 | 0.15 | 0.84 |
| Steel price | 4.00 | 0.53 | 0.13 | 0.82 |
| Conductor cross sectional area | 3.87 | 0.92 | 0.24 | 0.67 |
| Conductor erection length | 3.73 | 0.80 | 0.21 | 0.65 |
| Bored pile drilling length | 3.40 | 0.51 | 0.15 | 0.62 |
| Conductor unit price | 3.67 | 0.90 | 0.25 | 0.60 |
| Number of crossings | 3.53 | 1.06 | 0.30 | 0.51 |
| OPGW unit price | 3.20 | 0.68 | 0.21 | 0.49 |
| Earth excavation | 3.33 | 0.90 | 0.27 | 0.48 |
| Site grading | 3.13 | 0.74 | 0.24 | 0.45 |
| OPGW erection length | 3.27 | 0.96 | 0.29 | 0.43 |
| Number of tower nameplates | 3.07 | 0.80 | 0.26 | 0.40 |
| Tension insulator strings | 3.07 | 0.80 | 0.26 | 0.40 |
| Geological conditions | 2.53 | 0.74 | 0.29 | 0.21 |
| Climatic characteristics | 2.40 | 0.74 | 0.31 | 0.15 |
| Tower spacing | 2.53 | 1.06 | 0.42 | 0.08 |
| Crossing method | 2.27 | 0.80 | 0.35 | 0.07 |

### 3.2 Consistency Analysis

In this study, Kendall’s coefficient of concordance (Kendall’s W) was used to evaluate the consistency of expert ratings across the indicators. This method is suitable for measuring the degree of agreement among multiple raters when they evaluate the same set of items. It is especially appropriate for ordinal data and for assessing consensus among several evaluators. Kendall’s W was selected because it provides an unbiased measure of inter-rater agreement. It is suitable for the expert scoring context of this study and can account for subjective differences among different experts.

| N | Kendall’s W | Chi-square | df | Asymptotic significance |
| --- | --- | --- | --- | --- |
| 15 | .436 | 124.379 | 19 | <0.001 |

The results of Kendall’s W analysis show that the 15 experts reached a moderate level of agreement in their ratings of the evaluated indicators ((W = 0.436)). This indicates that the experts reached a consistent consensus in most cases. The chi-square value is 124.379, with 19 degrees of freedom, and the p-value is less than 0.001. This means that the observed agreement is statistically significant and is unlikely to be caused by random variation. Therefore, the ratings can be considered to show a relatively strong level of concordance among the experts.

### 3.3 Sensitivity Analysis

The composite score Pj is defined as follows:

Pj=α⋅μj'+1-α⋅CVj'

Where α=0.7. This study considers both the importance level of each indicator and the consistency of expert opinions. Sensitivity analysis was also conducted under different weighting combinations. The composite priority scores of all indicators were calculated and compared under (α = 0.5), 0.6, 0.7, and 0.8.

Table 4 Sensitivity Analysis

| Evaluation indicator | α=0.5 | α=0.6 | α=0.7 | α=0.8 |
| --- | --- | --- | --- | --- |
| Voltage level | 1.00 | 1.00 | 1.00 | 1.00 |
| Concrete volume | 0.91 | 0.91 | 0.91 | 0.91 |
| Foundation steel quantity | 0.82 | 0.83 | 0.85 | 0.86 |
| Tower steel quantity | 0.84 | 0.84 | 0.84 | 0.83 |
| Steel price | 0.85 | 0.83 | 0.82 | 0.80 |
| Conductor cross sectional area | 0.65 | 0.66 | 0.67 | 0.68 |
| Conductor erection length | 0.66 | 0.66 | 0.65 | 0.65 |
| Bored pile drilling length | 0.69 | 0.65 | 0.62 | 0.58 |
| Conductor unit price | 0.59 | 0.60 | 0.60 | 0.61 |
| Number of crossings | 0.47 | 0.49 | 0.51 | 0.52 |
| OPGW unit price | 0.55 | 0.52 | 0.49 | 0.47 |
| Earth excavation | 0.48 | 0.48 | 0.48 | 0.47 |
| Site grading | 0.49 | 0.47 | 0.45 | 0.42 |
| OPGW erection length | 0.42 | 0.43 | 0.43 | 0.43 |
| Number of tower nameplates | 0.44 | 0.42 | 0.40 | 0.39 |
| Tension insulator strings | 0.44 | 0.42 | 0.40 | 0.39 |
| Geological conditions | 0.26 | 0.23 | 0.21 | 0.18 |
| Climatic characteristics | 0.21 | 0.18 | 0.15 | 0.12 |
| Tower spacing | 0.06 | 0.07 | 0.08 | 0.09 |
| Crossing method | 0.11 | 0.09 | 0.07 | 0.04 |

### 3.4 Indicator Screening through Expert Review

Based on a comprehensive consideration of both indicator importance and expert consensus, four indicators with relatively low composite priority scores were removed. These indicators were Crossing method, Tower spacing, Geological conditions, and Climatic characteristics. These indicators were excluded for two main reasons. First, they had relatively low mean scores and greater disagreement among experts. Second, their effects were, to some extent, indirectly reflected by other design-stage variables, such as conductor cross-sectional area, steel tower material quantity, and number of crossings.

After expert screening, this study initially identified 16 modeling candidate indicators that can be directly quantified and are strongly associated with OHL cost.

## 4. Factor Analysis Results

After the 16 indicators were identified through expert screening, this study further applied exploratory factor analysis (EFA) to examine the latent structural relationships among the variables. This step was conducted to test the statistical rationality of the indicator system and reduce potential redundancy. Since this study focuses more on common variance than on principal component-based dimensionality reduction, principal axis factoring (PAF) was used for factor extraction. The suitability tests show that the Kaiser–Meyer–Olkin (KMO) value is 0.800. Bartlett’s test of sphericity gives an approximate chi-square value of 967.954, with (p < 0.001). These results indicate that the data have a good common factor structure and are suitable for factor analysis.

Because different dimensions of OHL cost are not completely independent in engineering practice, this study used oblique rotation rather than orthogonal rotation. For example, line scale may be related to foundation complexity, material quantities, and price levels at the same time. The pattern matrix was used as the main basis for assigning variables to factors. The results show that the 16 candidate variables can be summarized into four common factors.

Factor 1 mainly consists of earth excavation, site grading, conductor and OPGW erection length, and number of crossings. It reflects the dimension of line scale and on-site work quantities.

Factor 2 mainly consists of voltage level and conductor cross-sectional area. It reflects the dimension of technical grade and conductor selection.

Factor 3 mainly consists of conductor unit price and steel price. It reflects the dimension of material prices.

Factor 4 mainly consists of bored pile drilling length, foundation steel quantity, steel tower material quantity, concrete volume, and tension insulator string installation. It reflects the dimension of foundation complexity and component work quantities.

Table 5 KMO and Bartlett’s Test

| Kaiser-Meyer-Olkin Measure of Sampling Adequacy |  | .800 |
| --- | --- | --- |
| Bartlett’s Test of Sphericity | Approx. Chi-Square | 967.954 |
|  | df | 120 |
|  | Sig. | <.001 |

Table 6 Communalities

|  | Initial | Extraction |
| --- | --- | --- |
| Earth excavation | .972 | .943 |
| Site grading | .945 | .831 |
| Number of crossings | .921 | .787 |
| Conductor erection length | .906 | .845 |
| OPGW erection length | .971 | .942 |
| Voltage level | .797 | .995 |
| Conductor cross sectional area | .782 | .738 |
| OPGW unit price | .217 | .059 |
| Number of tower nameplates | .730 | .620 |
| Steel price | .519 | .428 |
| Conductor unit price | .708 | .937 |
| Bored pile drilling length | .933 | .825 |
| Foundation steel quantity | .991 | .992 |
| Tension insulator strings | .906 | .871 |

Extraction Method: Principal Axis Factoring

Fig 1 Scree plot

Table 7 Total Variance Explained

| Initial Eigenvalues |  |  |  | Extraction Sums of Squared Loadings |  |  | Rotation Sums of Squared Loadingsa |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Factor | Total | % of Variance | Cumulative % | Total | % of Variance | Cumulative % | Total |
| 1 | 9.139 | 57.119 | 57.119 | 9.023 | 56.391 | 56.391 | 7.148 |
| 2 | 1.752 | 10.949 | 68.068 | 1.591 | 9.946 | 66.337 | 2.261 |
| 3 | 1.495 | 9.341 | 77.409 | 1.215 | 7.596 | 73.933 | 3.125 |
| 4 | 1.081 | 6.759 | 84.168 | .944 | 5.902 | 79.835 | 7.232 |
| 5 | .938 | 5.863 | 90.032 |  |  |  |  |
| 6 | .443 | 2.768 | 92.799 |  |  |  |  |
| 7 | .363 | 2.268 | 95.067 |  |  |  |  |
| 8 | .231 | 1.442 | 96.509 |  |  |  |  |
| 9 | .162 | 1.010 | 97.519 |  |  |  |  |
| 10 | .151 | .946 | 98.465 |  |  |  |  |
| 11 | .110 | .686 | 99.151 |  |  |  |  |
| 12 | .071 | .444 | 99.595 |  |  |  |  |
| 13 | .035 | .216 | 99.811 |  |  |  |  |
| 14 | .016 | .102 | 99.913 |  |  |  |  |
| 15 | .010 | .062 | 99.975 |  |  |  |  |
| 16 | .004 | .025 | 100.000 |  |  |  |  |

Extraction Method: Principal Axis Factoring.

a. When factors are correlated, sums of squared loadings cannot be added to obtain a total variance.

Table8 Pattern Matrixa

|  | Factor |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Earth excavation | .952 |  |  |  |
| Site grading | .898 |  |  |  |
| OPGW erection length | .762 |  |  |  |
| Number of crossings | .762 |  |  |  |
| Conductor erection length | .751 |  |  |  |
| Number of tower nameplates |  |  |  |  |
| Voltage level |  | .945 |  |  |
| Conductor cross sectional area |  | .746 |  |  |
| OPGW unit price |  |  |  |  |
| Conductor unit price |  |  | .941 |  |
| Steel price |  |  | .623 |  |
| Bored pile drilling length |  |  |  | -.945 |
| Foundation steel quantity |  |  |  | -.849 |
| Tower steel quantity |  |  |  | -.757 |
| Tension insulator strings |  |  |  | -.751 |
| Concrete volume |  |  |  | -.743 |

Extraction Method: Principal Axis Factoring.

Rotation Method: Oblimin with Kaiser Normalization.

a. Rotation converged in 10 iterations.

Table A - 9 Structure Matrix

|  | Factor |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Earth excavation | .969 |  |  | -.589 |
| OPGW erection length | .937 |  |  | -.739 |
| Site grading | .911 |  |  | -.546 |
| Conductor erection length | .890 |  |  | -.685 |
| Number of crossings | .820 |  | .535 | -.515 |
| Voltage level |  | .970 |  |  |
| Conductor cross sectional area |  | .801 |  |  |
| OPGW unit price |  |  |  |  |
| Conductor unit price |  |  | .960 |  |
| Steel price |  |  | .644 |  |
| Foundation steel quantity | .713 |  |  | -.977 |
| Tower steel quantity | .752 |  |  | -.957 |
| Concrete volume | .779 |  |  | -.951 |
| Tension insulator strings | .686 |  |  | -.892 |
| Bored pile drilling length |  |  |  | -.877 |
| Number of tower nameplates | .617 |  |  | -.626 |

Extraction Method: Principal Axis Factoring.

Rotation Method: Oblimin with Kaiser Normalization.

Table A - 10 Factor Correlation Matrix

| Factor | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| 1 | 1.000 | .128 | .290 | -.584 |
| 2 | .128 | 1.000 | .035 | -.220 |
| 3 | .290 | .035 | 1.000 | -.415 |
| 4 | -.584 | -.220 | -.415 | 1.000 |

Extraction Method: Principal Axis Factoring.

Rotation Method: Oblimin with Kaiser Normalization.

Further analysis shows that OPGW unit price does not produce a significant pattern loading on any common factor. It also has relatively low communality. This indicates that it shares limited common variance with the other variables. Therefore, OPGW unit price was finally removed from the variable set.

In contrast, the number of tower nameplates does not form a typical common-factor cluster. However, in engineering terms, it can reflect line segmentation density and the complexity of corridor organization. Therefore, it was retained as an independent contextual variable for subsequent modeling.

As a result, a total of 15 input variables were finally used for model training.